# U-Boot Details and How Linux board get Bootup

# UBoot Concept

# Download uboot from official link

# Link: 	https://ftp.denx.de/pub/u-boot/

# https://github.com/u-boot/u-boot

# Go to SOC folder

# cd board

# cd ti

# Here am335x folder is available which use in beagle bone black

# Cpu specific folder and files (CPU based initialization for different Architecture)

# cd arch/arm

# cd cpu

# cd armv7  (It is for beaglebone black)

# RBL (ROM Bootloader) It is inside SOC

# Start.s

# SPL      CPU Inits

# Soc Inits            Read and Jump to uBoot

# SPL  Secondary Program Loader

# -------------------------------------------------------------------------------------------------------

# AM335x ROM

# ↓ Loads

# MLO (contains SPL) Here MLO is file type and SPL is code

# ↓ loads

# u-boot.img

# ↓ loads

# Linux Kernel

#   SPL = the first-stage bootloader program.

#   MLO = the boot image/file containing SPL for TI devices.

# Sequence of Booting

# MLO

# u-boot.img

# zImage

# am335x-boneblack.dtb

# ***********************************************************************

| # Feature | # LKM | # Device Tree Overlay |
| --- | --- | --- |
| # Contains executable code | # Yes | # No |
| # Contains hardware description | # No | # Yes |
| # File type | # .ko | # .dtbo |
| # Loaded at runtime | # Yes | # Yes |
| # Used for drivers | # Yes | # No |
| # Used for pinmux/hardware config | # No | # Yes |

### ---------------------------------------------------------------------------------------------------              BeagleBone Black Example

Suppose you connect an SPI LCD.

Overlay (.dtbo):

Enable SPI0
Configure pins
Describe LCD

LKM (.ko):

LCD driver code
Send commands
Draw pixels

So the relationship is:

Overlay (.dtbo)
        +
Driver (.ko)
        ↓
Working hardware